|  | RESULT REPORTING SOP Post-Analytical Standard Operating Procedure — CLIA §493.1291 | SOP #: SOP-RPT-001 Version: 1.0 Effective: ________ Review Due: ________ |
| --- | --- | --- |

1. PURPOSE & SCOPE

This SOP defines the procedures for accurate, timely, and complete reporting of laboratory test results at Lab2Doctors. It applies to all laboratory personnel involved in result verification, authorization, and release via the Laboratory Information System (LIS) or manual reporting pathways.

2. REGULATORY BASIS

| Regulation / Standard | Requirement |
| --- | --- |
| CLIA 42 CFR §493.1291 | Results must include patient ID, test name, result, reference interval, and collection date/time |
| CLIA 42 CFR §493.1291(h) | Critical values must be immediately reported to authorized person; documentation required |
| CAP COM.04050 | Turnaround time goals must be defined, monitored, and met |
| HIPAA §164.514 | Results must be transmitted only to authorized recipients |
| CAP GEN.41096 | Amended reports must be clearly identified as such |

3. REQUIRED ELEMENTS ON EVERY RESULT REPORT

| # | Required Element | Notes / Standards |
| --- | --- | --- |
| 1 | Patient full name and date of birth | Must match two identifiers used at collection |
| 2 | Patient Medical Record Number (MRN) or accession number | Unique identifier for traceability |
| 3 | Name and address of laboratory performing the test | Required for send-out / reference lab results |
| 4 | Test name (full name — no abbreviations for patient-facing reports) | Use standard LOINC nomenclature in LIS |
| 5 | Specimen type and collection date/time | Critical for timed tests and stability assessment |
| 6 | Numerical result with units | SI units preferred per CLSI; flag if no result |
| 7 | Reference interval (age- and sex-specific where applicable) | Lab-verified reference intervals per CLIA §493.1253 |
| 8 | Abnormal flag (H/L/Critical) where applicable | Apply per validated reference intervals |
| 9 | Name and credentials of authorizing laboratory director or designee | Electronically signed in LIS |
| 10 | Date and time of result release | Auto-generated by LIS upon verification |

4. RESULT VERIFICATION WORKFLOW

| Step | Action | Responsible Party / Details |
| --- | --- | --- |
| 1 | Instrument result reviewed in LIS | CLS verifies result plausibility; checks QC status for the run |
| 2 | Delta check performed | Compare to prior patient result per Delta Check Criteria Sheet (DCK-F-001) |
| 3 | Critical value check | Compare result against Critical Values List (CVL-F-001); if critical — go to Step 4 |
| 4 | Critical value notification | Notify ordering clinician within defined TAT (see Critical Values List); document in Critical Result Log |
| 5 | Reference interval flagging | LIS auto-applies H/L flags; CLS confirms flags are appropriate for patient demographics |
| 6 | Result authorization / verification | CLS or Technical Supervisor electronically verifies result in LIS |
| 7 | Result transmission | LIS auto-transmits to EHR / physician portal / fax per client setup |
| 8 | Amended result (if needed) | Follow Amended Report Procedure below; original and amended both retained |

5. AMENDED REPORT PROCEDURE

An amended report is required when a previously released result is changed for any reason (transcription error, dilution repeat, specimen identity correction, etc.).

| Step | Action | Documentation Required |
| --- | --- | --- |
| 1 | Identify need for amendment | Document reason: error type, discovery method |
| 2 | Notify ordering clinician immediately if result change is clinically significant | Document name, time, and method of notification |
| 3 | Enter amended result in LIS; mark clearly as AMENDED | Include original result, amended result, reason, and authorizing person |
| 4 | Original report retained in LIS permanently alongside amended report | Both accessible for audit; never delete original |
| 5 | Log in NCR/incident system if error was laboratory-caused | Assign CAPA if systemic issue identified |

6. TURNAROUND TIME (TAT) TARGETS

| Test Category | Priority | TAT Target | Measured From → To |
| --- | --- | --- | --- |
| STAT Chemistry / CBC | STAT | ≤ 60 minutes | Order receipt → Result verified |
| Routine Chemistry / CBC | Routine | ≤ 2 hours | Order receipt → Result verified |
| Microbiology (Gram stain) | STAT | ≤ 60 minutes | Specimen receipt → Result reported |
| Blood Culture (preliminary) | Routine | ≤ 5 days | Instrument flag → Physician notification |
| Coagulation (PT/INR) | STAT | ≤ 45 minutes | Order receipt → Result verified |
| Critical Values Notification | Any | ≤ 30 minutes | Result verified → Clinician contacted |

7. APPROVAL & REVISION HISTORY

| Version | Summary of Changes | Author | Approval Date |
| --- | --- | --- | --- |
| 1.0 | Initial release |  |  |

| Technical Supervisor | Quality Manager | Laboratory Director |
| --- | --- | --- |
| Signature: _______________ Name: ___________________ Date: ___________________ | Signature: _______________ Name: ___________________ Date: ___________________ | Signature: _______________ Name: ___________________ Date: ___________________ |